Q. Demonstrate FTP Server in Packet Tracer.

Aim: To configure an FTP server and test file transfer between server and clients in the network.

Devices Needed:

1 Server (FTP Server)

- Switch

- PCs (FTP Clients) Connection :

Connect all devices to the switch using copper straight-through cables

Topology:

Code:

Server Configuration (FTP):

1.Click on Server → Services tab → FTP

2.FTP Server Settings: oFTP Service: ON

3.Add User Account: oUsername: admin oPassword: cisco

oCheck all permissions: Read/Write/List/Delete/Rename

oClick Add

Server IP Configuration:

IP Address: 192.168.1.5

Subnet Mask: 255.255.255.0 Default Gateway: 192.168.1.1 PC1 Configuration:

IP Address: 192.168.1.10

Subnet Mask: 255.255.255.0 Default Gateway: 192.168.1.1 PC2 Configuration:

IP Address: 192.168.1.11

Subnet Mask: 255.255.255.0

Default Gateway: 192.168.1.1

Create Test File on PC1

1.Click on PC1 → Desktop tab → Text Editor

2.Type: This is a test file for FTP transfer

3.Save as: upload_test.txt Verification: In PC Command Prompt PC1 FTP Commands:

C:\>ftp 192.168.1.5

Username: admin Password: cisco ftp> dir ftp> put upload_test.txt ftp> dir ftp> get welcome.txt ftp> delete notes.txt ftp> quit

PC2 FTP Commands:

C:\>ftp 192.168.1.5

Username: admin Password: cisco ftp> dir ftp> get upload_test.txt ftp> rename upload_test.txt downloaded_file.txt ftp> dir ftp> quit

Ping Test (Both PCs):

C:\>ping 192.168.1.5

PC1 FTP Session:

Output:

Ping Test Results:

PC2 FTP Session:

Ping Test Results:

Conclusion: The FTP server has been configured successfully